# Rover External Control Node (rov_ext_cpp)

## Overview

This ROS 2 package implements rov_ext, an external control node for PX4 Rovers.
It performs:

- PX4 External Mode registration (self-managed, no duplicate ModeBase registration).

- Continuous config heartbeat publishing (/fmu/in/config_control_setpoints).

- Rover throttle + steering setpoints publishing (/fmu/in/rover_*_setpoint) when enabled.

- Optional goto keep-alive publishing.

- Mirrors source_id from PX4 (/fmu/out/vehicle_control_mode) to stay in sync with whichever External slot PX4 assigns.

This prevents mismatches, duplicate registrations, and “safe recovery” triggers.

## Package Layout

ros2_ws/src/rov_ext/

├── CMakeLists.txt

├── package.xml

├── include/

│    └── mode.hpp        # Main node class

└── src/

└── main.cpp        # Entry point

## package.xml

<?xml version="1.0"?>

<?xml-model href="http://download.ros.org/schema/package_format3.xsd" schematypens="http://www.w3.org/2001/XMLSchema"?>

<package format="3">

<name>rov_ext_cpp</name>

<version>0.0.1</version>

<description>Rover External Control (rov_ext): PX4 external mode heartbeat + rover setpoints</description>

<maintainer email="vinothpandiyan@hotmail.com">Vinoth Pandiyan</maintainer>

<license>BSD-3-Clause</license>

<buildtool_depend>eigen3_cmake_module</buildtool_depend>

<buildtool_depend>ament_cmake</buildtool_depend>

<buildtool_export_depend>eigen3_cmake_module</buildtool_export_depend>

<build_depend>eigen</build_depend>

<build_depend>rclcpp</build_depend>

<build_export_depend>eigen</build_export_depend>

<depend>px4_ros2_cpp</depend>

<test_depend>ament_lint_auto</test_depend>

<test_depend>ament_lint_common</test_depend>

<export>

<build_type>ament_cmake</build_type>

</export>

</package>

## CMakeLists.txt

cmake_minimum_required(VERSION 3.5)

project(rov_ext_cpp)

if(NOT CMAKE_CXX_STANDARD)

set(CMAKE_CXX_STANDARD 17)

endif()

if(CMAKE_COMPILER_IS_GNUCXX OR CMAKE_CXX_COMPILER_ID MATCHES "Clang")

add_compile_options(-Wall -Wextra -Wpedantic -Werror -Wno-unused-parameter)

endif()

set(CMAKE_EXPORT_COMPILE_COMMANDS ON)

find_package(eigen3_cmake_module REQUIRED)

find_package(Eigen3 REQUIRED)

find_package(ament_cmake REQUIRED)

find_package(rclcpp REQUIRED)

find_package(px4_ros2_cpp REQUIRED)

include_directories(include ${Eigen3_INCLUDE_DIRS})

add_executable(rov_ext

src/main.cpp

)

target_link_libraries(rov_ext PUBLIC

Eigen3::Eigen

px4_ros2_cpp::px4_ros2_cpp

rclcpp::rclcpp

)

install(TARGETS

rov_ext

DESTINATION lib/${PROJECT_NAME}

)

if(BUILD_TESTING)

find_package(ament_lint_auto REQUIRED)

ament_lint_auto_find_test_dependencies()

endif()

ament_package()

## main.cpp

#include "rclcpp/rclcpp.hpp"

#include <px4_ros2/components/node_with_mode.hpp>

#include <string>

#include "../include/mode.hpp"

// Wrap our ModeBase class so rclcpp::spin works

using MyNodeWithMode = px4_ros2::NodeWithMode<FlightModeTest>;

static const std::string kNodeName = "rov_ext";   // ROS node name

static const bool kEnableDebugOutput = true;      // Enable ModeBase debug

int main(int argc, char* argv[])

{

rclcpp::init(argc, argv);

rclcpp::spin(std::make_shared<MyNodeWithMode>(kNodeName, kEnableDebugOutput));

rclcpp::shutdown();

return 0;

}

include/mode.hpp (documented)

#pragma once

/****************************************************************************

* rov_ext — Rover External Control (PX4 + ROS 2)

*

* WHAT THIS NODE DOES

* - Registers one External component with PX4 (self-registration only).

* - Publishes a "control config heartbeat" on /fmu/in/config_control_setpoints:

*     * AUTO + ALLOCATION enabled

*     * all other controller flags disabled (no MC loops)

* - Publishes rover setpoints (throttle/steering) when `enabled=true`.

* - Optionally publishes a GotoSetpoint “hold” (0 speed) keepalive.

* - Mirrors PX4’s active External slot `source_id` by subscribing to

*   /fmu/out/vehicle_control_mode so the heartbeat always matches FMU.

*

* WHY SELF-REGISTRATION ONLY?

* - Avoids double registration (ModeBase + self), which leads to multiple

*   External slots (e.g., External 1 & 2) and QGC confusion.

* - You can flip to ModeBase registration later if you want, but this base

*   keeps a single registrar to stay deterministic.

*

* KEY TOPICS

*   IN:

*     /fmu/in/config_control_setpoints   (px4_msgs/VehicleControlMode)

*     /fmu/in/rover_throttle_setpoint    (px4_msgs/RoverThrottleSetpoint)

*     /fmu/in/rover_steering_setpoint    (px4_msgs/RoverSteeringSetpoint)

*     /fmu/in/goto_setpoint              (px4_msgs/GotoSetpoint) [optional]

*     /fmu/in/register_ext_component_request (px4_msgs/RegisterExtComponentRequest)

*     /fmu/in/vehicle_command            (px4_msgs/VehicleCommand)

*

*   OUT:

*     /fmu/out/vehicle_control_mode      (mirror source_id + status)

*     /fmu/out/register_ext_component_reply (registration ack)

*     /fmu/out/arming_check_request      (debug-only)

*

* CONTROL LOOP RATES (defaults; configurable via ROS params)

*   heartbeat_hz = 1   (recommend 2–5 for robustness)

*   setpoint_hz  = 20  (rover setpoints when enabled=true)

*

* SAFE OPERATION CHECKLIST

* - Ensure exactly ONE registrar is active (this file does that by default).

* - Keep a continuous signal in External mode:

*     * set `enabled=true` (even with zero throttle/steer)

*     * or `use_goto_keepalive=true` to send a 0-speed hold

* - Consider setting heartbeat_hz >= 2 to avoid “control signal lost”.

****************************************************************************/

#include <px4_ros2/components/mode.hpp>

#include <px4_ros2/odometry/local_position.hpp>

#include <px4_ros2/control/setpoint_types/experimental/rates.hpp>

#include <px4_ros2/control/setpoint_types/experimental/attitude.hpp>

#include <px4_msgs/msg/vehicle_control_mode.hpp>

#include <px4_msgs/msg/register_ext_component_request.hpp>

#include <px4_msgs/msg/register_ext_component_reply.hpp>

#include <px4_msgs/msg/arming_check_request.hpp>

#include <px4_msgs/msg/vehicle_command.hpp>

#include <px4_msgs/msg/rover_throttle_setpoint.hpp>

#include <px4_msgs/msg/rover_steering_setpoint.hpp>

#include <px4_msgs/msg/goto_setpoint.hpp>

#include <rclcpp/rclcpp.hpp>

#include <algorithm>

#include <cstring>

#include <string>

#include <chrono>

#include <limits>

using namespace std::chrono_literals;

// Name shown by ModeBase in logs; the QGC external name uses `component_name` param below.

static const std::string kModeName = "Rover_External_Control";

static inline float clampf(float v, float lo, float hi) {

return std::min(std::max(v, lo), hi);

}

class FlightModeTest : public px4_ros2::ModeBase

{

public:

explicit FlightModeTest(rclcpp::Node & node)

: ModeBase(node, kModeName), node_(&node)

{

// Be tolerant to minor message definition drift across versions

this->setSkipMessageCompatibilityCheck();

// ---------- Parameters ----------

// External slot hint. Actual selection is decided by PX4; we mirror later.

external_slot_   = declare_param<int>("external_slot", 1);

source_id_       = (external_slot_ == 2) ? 24u : 23u; // (External1=23, External2=24, External3=25...)

heartbeat_hz_    = std::max<int>(1, declare_param<int>("heartbeat_hz", 1));

setpoint_hz_     = std::max<int>(1, declare_param<int>("setpoint_hz", 20));

// REGISTRATION POLICY: self-registration ONLY by default.

// (Prevents double registration; QGC shows a single external mode.)

do_register_     = declare_param<bool>("do_register", true);     // we will send one registration

reg_arm_check_   = declare_param<bool>("register_arming_check", false); // disable ModeBase auto-reg

reg_mode_        = declare_param<bool>("register_mode", false);

reg_mode_exec_   = declare_param<bool>("register_mode_executor", false);

// Short, user-facing name for QGC (external mode list)

comp_name_       = declare_param<std::string>("component_name", "rov_ext");

// Rover setpoints publishing toggle + optional keepalive

enabled_         = declare_param<bool>("enabled", false);

use_goto_keepalive_ = declare_param<bool>("use_goto_keepalive", false);

// Rover command inputs (normalized in [-1, +1])

throttle_x_      = clampf((float)declare_param<double>("throttle_body_x", 0.0), -1.f, 1.f);

throttle_y_      = clampf((float)declare_param<double>("throttle_body_y", 0.0), -1.f, 1.f); // skid steer only

steering_        = clampf((float)declare_param<double>("steering_angle",   0.0), -1.f, 1.f);

speed_diff_      = clampf((float)declare_param<double>("speed_diff",       0.0), -1.f, 1.f); // skid steer only

// Live parameter updates

param_cb_ = node_->add_on_set_parameters_callback(

[this](const std::vector<rclcpp::Parameter> &ps) {

for (const auto &p : ps) {

const auto &n = p.get_name();

if (n == "external_slot")       { external_slot_ = p.as_int(); /* hint only; FMU decides */ }

else if (n == "heartbeat_hz")   { heartbeat_hz_ = std::max<int>(1, (int)p.as_int());  if (cfg_timer_)  cfg_timer_->reset(); }

else if (n == "setpoint_hz")    { setpoint_hz_  = std::max<int>(1, (int)p.as_int());  if (tick_timer_) tick_timer_->reset(); }

else if (n == "do_register")    { do_register_  = p.as_bool(); } // send happens in onActivate()

else if (n == "register_arming_check")  { reg_arm_check_ = p.as_bool(); }

else if (n == "register_mode")          { reg_mode_ = p.as_bool(); }

else if (n == "register_mode_executor") { reg_mode_exec_ = p.as_bool(); }

else if (n == "component_name") { comp_name_ = p.as_string(); }

else if (n == "enabled")        { enabled_ = p.as_bool(); }

else if (n == "use_goto_keepalive") { use_goto_keepalive_ = p.as_bool(); }

else if (n == "throttle_body_x"){ throttle_x_ = clampf(p.as_double(), -1.0, 1.0); }

else if (n == "throttle_body_y"){ throttle_y_ = clampf(p.as_double(), -1.0, 1.0); }

else if (n == "steering_angle") { steering_   = clampf(p.as_double(), -1.0, 1.0); }

else if (n == "speed_diff")     { speed_diff_ = clampf(p.as_double(), -1.0, 1.0); }

}

rcl_interfaces::msg::SetParametersResult r; r.successful = true; return r;

});

// ---------- Publishers / Subscribers ----------

auto be = rclcpp::QoS(10).best_effort(); // uXRCE PX4 expects best_effort on /fmu/in

cfg_pub_  = node_->create_publisher<px4_msgs::msg::VehicleControlMode>    ("/fmu/in/config_control_setpoints", be);

thr_pub_  = node_->create_publisher<px4_msgs::msg::RoverThrottleSetpoint> ("/fmu/in/rover_throttle_setpoint",  be);

str_pub_  = node_->create_publisher<px4_msgs::msg::RoverSteeringSetpoint> ("/fmu/in/rover_steering_setpoint",  be);

goto_pub_ = node_->create_publisher<px4_msgs::msg::GotoSetpoint>          ("/fmu/in/goto_setpoint",            be);

// Registration I/O (self-registration path)

reg_pub_  = node_->create_publisher<px4_msgs::msg::RegisterExtComponentRequest>("/fmu/in/register_ext_component_request", 10);

reg_sub_  = node_->create_subscription<px4_msgs::msg::RegisterExtComponentReply>(

"/fmu/out/register_ext_component_reply", rclcpp::QoS(10).best_effort(),

[this](px4_msgs::msg::RegisterExtComponentReply::SharedPtr){

RCLCPP_INFO(node_->get_logger(),"Got RegisterExtComponentReply");

});

// Arm-check request (informational)

armchk_sub_ = node_->create_subscription<px4_msgs::msg::ArmingCheckRequest>(

"/fmu/out/arming_check_request", rclcpp::QoS(10).best_effort(),

[this](px4_msgs::msg::ArmingCheckRequest::SharedPtr m){

if (!printed_armchk_) { printed_armchk_=true; RCLCPP_DEBUG(node_->get_logger(),"Arming check request (id=%u, only printed once)", m->request_id); }

});

// VehicleCommand publisher (for arm/disarm convenience if you need it)

cmd_pub_  = node_->create_publisher<px4_msgs::msg::VehicleCommand>("/fmu/in/vehicle_command", 10);

// Mirror FMU's active source_id → guarantees heartbeat matches PX4’s chosen slot

vcm_sub_ = node_->create_subscription<px4_msgs::msg::VehicleControlMode>(

"/fmu/out/vehicle_control_mode", rclcpp::QoS(10).best_effort(),

[this](const px4_msgs::msg::VehicleControlMode::SharedPtr m)

{

if (m->source_id != source_id_) {

source_id_ = m->source_id; // 0 until assigned; then 23/24/25...

if (!printed_source_warn_) {

printed_source_warn_ = true;

RCLCPP_WARN(node_->get_logger(),

"FMU active slot source_id=%u; mirroring to match (prevents mismatch).",

source_id_);

}

}

});

// ---------- Timers ----------

tick_timer_ = node_->create_wall_timer(

std::chrono::milliseconds(1000/std::max(1,setpoint_hz_)),

std::bind(&FlightModeTest::tick,this));

cfg_timer_  = node_->create_wall_timer(

std::chrono::milliseconds(1000/std::max(1,heartbeat_hz_)),

std::bind(&FlightModeTest::publishConfig,this));

// Optional local position helper for goto keepalive

lpos_ = std::make_shared<px4_ros2::OdometryLocalPosition>(*this);

// Dummy setpoint types required by ModeBase (we do not publish MC setpoints)

_rates_sp_ = std::make_shared<px4_ros2::RatesSetpointType>(*this);

_att_sp_   = std::make_shared<px4_ros2::AttitudeSetpointType>(*this);

}

void onActivate() override

{

RCLCPP_DEBUG(node_->get_logger(), "Registering '%s' (arming check: %d, mode: %d, mode executor: %d)",

comp_name_.c_str(), reg_arm_check_, reg_mode_, reg_mode_exec_);

// Startup burst to help PX4 latch config (esp. during timesync)

burst_left_ = 10;

burst_timer_ = node_->create_wall_timer(50ms, std::bind(&FlightModeTest::burstTick, this));

// Self-registration (single registrar)

if (do_register_) {

sendRegistration();

} else {

RCLCPP_WARN(node_->get_logger(), "Skipping registration (param do_register=false)");

}

last_cfg_log_ns_ = 0;

RCLCPP_DEBUG(node_->get_logger(), "Mode '%s' activated", kModeName.c_str());

}

void onDeactivate() override {}

// No multicopter publishing → keeps all MC loops off

void updateSetpoint(float) override { /* no-op */ }

private:

template<typename T> T declare_param(const std::string &name, const T &def){

return node_->declare_parameter<T>(name, def);

}

// Send exactly one RegisterExtComponentRequest (self-registration)

void sendRegistration()

{

px4_msgs::msg::RegisterExtComponentRequest req{};

req.timestamp = 0;

std::fill(req.name.begin(), req.name.end(), 0);

const size_t n = std::min(comp_name_.size(), req.name.size()-1);

std::memcpy(req.name.data(), comp_name_.data(), n);

req.register_arming_check  = reg_arm_check_;   // false (we’re not using ModeBase registrar)

req.register_mode          = reg_mode_;        // false

req.register_mode_executor = reg_mode_exec_;   // false

reg_pub_->publish(req);

}

// Heartbeat: enable AUTO + ALLOCATION, keep other loops disabled

void publishConfig()

{

px4_msgs::msg::VehicleControlMode m{};

m.timestamp = 0;

m.flag_armed = false;

m.flag_multicopter_position_control_enabled = false;

m.flag_control_manual_enabled       = false;

m.flag_control_auto_enabled         = true;

m.flag_control_offboard_enabled     = false;

m.flag_control_position_enabled     = false;

m.flag_control_velocity_enabled     = false;

m.flag_control_altitude_enabled     = false;

m.flag_control_climb_rate_enabled   = false;

m.flag_control_acceleration_enabled = false;

m.flag_control_attitude_enabled     = false;

m.flag_control_rates_enabled        = false;

m.flag_control_allocation_enabled   = true;

m.flag_control_termination_enabled  = false;

// CRITICAL: publish the FMU’s active slot (mirrored) to avoid mismatches

m.source_id = source_id_;

cfg_pub_->publish(m);

const auto now = node_->get_clock()->now().nanoseconds();

if (now - last_cfg_log_ns_ > 5'000'000'000LL) {

last_cfg_log_ns_ = now;

RCLCPP_INFO(node_->get_logger(), "External conductor: config heartbeat sent (source_id=%u)", source_id_);

}

}

// Optional keepalive: hold position with 0 speed to show “active control”

void publishGotoKeepAlive()

{

if (!use_goto_keepalive_) return;

const auto ned = lpos_->positionNed();

px4_msgs::msg::GotoSetpoint g{};

g.timestamp = 0;

g.position[0]=ned.x(); g.position[1]=ned.y(); g.position[2]=ned.z();

g.flag_control_heading=false; g.heading=std::numeric_limits<float>::quiet_NaN();

g.flag_set_max_horizontal_speed=true; g.max_horizontal_speed=0.0f;

g.flag_set_max_vertical_speed=false;  g.max_vertical_speed  =0.0f;

g.flag_set_max_heading_rate=true;     g.max_heading_rate    =0.7854f;

goto_pub_->publish(g);

}

// Rover setpoints (published at setpoint_hz when `enabled=true`)

void publishRoverSetpoints()

{

if (!enabled_) return;

px4_msgs::msg::RoverThrottleSetpoint t{}; t.timestamp=0;

t.throttle_body_x=throttle_x_; t.throttle_body_y=throttle_y_;

px4_msgs::msg::RoverSteeringSetpoint s{}; s.timestamp=0;

s.normalized_steering_angle=steering_; s.normalized_speed_diff=speed_diff_;

thr_pub_->publish(t); str_pub_->publish(s);

}

void burstTick()

{

if (burst_left_-- > 0) { publishConfig(); if (enabled_) publishRoverSetpoints(); }

else { burst_timer_->cancel(); }

}

void tick()

{

publishRoverSetpoints();

publishGotoKeepAlive();

}

private:

rclcpp::Node* node_{nullptr};

// --------- Params / state ---------

int external_slot_{1};                    // hint only

unsigned source_id_{23};                  // mirrored from FMU at runtime

int heartbeat_hz_{1};

int setpoint_hz_{20};

// Self-registration ONLY (defaults aligned with constructor)

bool do_register_{true};

bool reg_arm_check_{false};

bool reg_mode_{false};

bool reg_mode_exec_{false};

std::string comp_name_{"rov_ext"};

bool enabled_{false};

bool use_goto_keepalive_{false};

// Command inputs

float throttle_x_{0.f}, throttle_y_{0.f}, steering_{0.f}, speed_diff_{0.f};

// --------- ROS I/O ---------

rclcpp::Publisher<px4_msgs::msg::VehicleControlMode>::SharedPtr          cfg_pub_;

rclcpp::Publisher<px4_msgs::msg::RoverThrottleSetpoint>::SharedPtr       thr_pub_;

rclcpp::Publisher<px4_msgs::msg::RoverSteeringSetpoint>::SharedPtr       str_pub_;

rclcpp::Publisher<px4_msgs::msg::GotoSetpoint>::SharedPtr                goto_pub_;

rclcpp::Publisher<px4_msgs::msg::RegisterExtComponentRequest>::SharedPtr reg_pub_;

rclcpp::Subscription<px4_msgs::msg::RegisterExtComponentReply>::SharedPtr reg_sub_;

rclcpp::Subscription<px4_msgs::msg::ArmingCheckRequest>::SharedPtr       armchk_sub_;

rclcpp::Publisher<px4_msgs::msg::VehicleCommand>::SharedPtr              cmd_pub_;

// Mirror FMU source_id

rclcpp::Subscription<px4_msgs::msg::VehicleControlMode>::SharedPtr vcm_sub_;

bool printed_source_warn_{false};

// Helpers / timers

std::shared_ptr<px4_ros2::OdometryLocalPosition> lpos_;

rclcpp::TimerBase::SharedPtr tick_timer_, cfg_timer_, burst_timer_;

int burst_left_{0};

int64_t last_cfg_log_ns_{0};

bool printed_armchk_{false};

// Required by ModeBase (not used to publish MC setpoints)

std::shared_ptr<px4_ros2::RatesSetpointType>    _rates_sp_;

std::shared_ptr<px4_ros2::AttitudeSetpointType> _att_sp_;

rclcpp::node_interfaces::OnSetParametersCallbackHandle::SharedPtr param_cb_;

};

Run & Test

Build:

cd ~/ros2_ws

colcon build --packages-select rov_ext_cpp --cmake-clean-cache

source install/setup.bash

Run:

ros2 run rov_ext_cpp rov_ext

Verify:

# 1 registrar

ros2 topic info -v /fmu/in/register_ext_component_request

# PX4’s slot (mirrored)

ros2 topic echo --once /fmu/out/vehicle_control_mode

# Enable rover output

ros2 param set /rov_ext enabled true

ros2 param set /rov_ext throttle_body_x 0.5

ros2 param set /rov_ext steering_angle 0.2

🚘 Rover External Control (rov_ext)

A ROS 2 package providing an External Mode control node for PX4 Rovers.
The node (rov_ext) handles PX4 external mode registration, config heartbeat, and rover setpoints with safe slot mirroring.

✨ Features

- Self-registers once as a PX4 External Mode component

- Publishes config heartbeat → /fmu/in/config_control_setpoints

- Publishes rover setpoints (throttle + steering) when enabled

- Optional goto keep-alive setpoints → /fmu/in/goto_setpoint

- Mirrors PX4’s assigned source_id from /fmu/out/vehicle_control_mode

  - Prevents mismatches and duplicate external slots

- Fully parameterized via ROS 2 dynamic parameters

📂 Package Structure

rov_ext_cpp/

├── CMakeLists.txt

├── package.xml

├── include/

│    └── mode.hpp      # Core node implementation

├── src/

│    └── main.cpp      # Entry point

└── README.md

⚙️ Build Instructions

From your ROS 2 workspace root (~/ros2_ws):

colcon build --packages-select rov_ext_cpp --cmake-clean-cache

source install/setup.bash

▶️ Run the Node

ros2 run rov_ext_cpp rov_ext

🔧 Parameters

All parameters can be updated at runtime with ros2 param set.

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| enabled | bool | false | Enable rover throttle/steering publishing |
| use_goto_keepalive | bool | false | Enable keep-alive goto setpoints (zero velocity hold) |
| heartbeat_hz | int | 5 | Frequency of config heartbeat (Hz) |
| setpoint_hz | int | 20 | Frequency of rover setpoint publishing (Hz) |
| throttle_body_x | double | 0.0 | Normalized throttle [-1..1] |
| steering_angle | double | 0.0 | Normalized steering [-1..1] |
| speed_diff | double | 0.0 | Speed differential (skid-steer) |
| component_name | string | rov_ext | Component name shown in PX4/QGC |

📡 Topics

Published

- /fmu/in/config_control_setpoints → px4_msgs/msg/VehicleControlMode

- /fmu/in/rover_throttle_setpoint → px4_msgs/msg/RoverThrottleSetpoint

- /fmu/in/rover_steering_setpoint → px4_msgs/msg/RoverSteeringSetpoint

- /fmu/in/goto_setpoint → px4_msgs/msg/GotoSetpoint

Subscribed

- /fmu/out/vehicle_control_mode → mirrors PX4’s active source_id

🧪 Testing

- Enable rover output:

- ros2 param set /rov_ext enabled true

- ros2 param set /rov_ext throttle_body_x 0.3

- ros2 param set /rov_ext steering_angle 0.1

- Check config heartbeat:

- ros2 topic echo --once /fmu/in/config_control_setpoints

- Check PX4 slot mirroring:

- ros2 topic echo --once /fmu/out/vehicle_control_mode

- # → source_id matches PX4’s active External slot (23, 24, etc.)

👤 Maintainer

- Vinoth Pandiyan
📧 vinothpandiyan@hotmail.com