#pragma once

/****************************************************************************

* PX4 External Mode — Quiet "Conductor" for Rover (Ackermann)

*

* Keeps your example_mode_manual binary.

* - Publishes legacy control-config heartbeat (External + allocation).

* - Hints External-2 via source_id=24.

* - NO multicopter setpoints (updateSetpoint is a NO-OP).

* - Optional rover throttle/steer outputs when ENABLE_ROVER_OUTPUTS=1.

****************************************************************************/

#ifndef ENABLE_ROVER_OUTPUTS

#define ENABLE_ROVER_OUTPUTS 0   // 0 = conductor-only (no throttle/steer publishes)

#endif

#include <px4_ros2/components/mode.hpp>

#include <px4_ros2/control/setpoint_types/experimental/rates.hpp>

#include <px4_ros2/control/setpoint_types/experimental/attitude.hpp>

#include <px4_ros2/control/peripheral_actuators.hpp>

#include <px4_ros2/utils/geometry.hpp>

#include <px4_ros2/odometry/local_position.hpp>

#include <px4_msgs/msg/rover_throttle_setpoint.hpp>

#include <px4_msgs/msg/rover_steering_setpoint.hpp>

#include <px4_msgs/msg/vehicle_control_mode.hpp>   // legacy config IN

#include <px4_msgs/msg/vehicle_command.hpp>

#include <px4_msgs/msg/goto_setpoint.hpp>

#include <Eigen/Eigen>

#include <rclcpp/rclcpp.hpp>

#include <algorithm>

#include <chrono>

#include <cmath>

#include <cstdint>

#include <limits>

#include <memory>

#include <string>

using namespace std::chrono_literals;

static const std::string kName = "My Manual Mode";

static inline float clampf(float v, float lo, float hi) {

return std::min(std::max(v, lo), hi);

}

class FlightModeTest : public px4_ros2::ModeBase

{

public:

explicit FlightModeTest(rclcpp::Node & node)

: ModeBase(node, kName), node_(&node)

{

this->setSkipMessageCompatibilityCheck(); // tolerate schema drift

// --- default helpers kept (we won't publish MC setpoints) ---

_manual_control_input         = std::make_shared<px4_ros2::ManualControlInput>(*this);

_rates_setpoint               = std::make_shared<px4_ros2::RatesSetpointType>(*this);

_attitude_setpoint            = std::make_shared<px4_ros2::AttitudeSetpointType>(*this);

_peripheral_actuator_controls = std::make_shared<px4_ros2::PeripheralActuatorControls>(*this);

// --- rover publishers ---

auto qos_be = rclcpp::QoS(10).best_effort(); // XRCE subscribers expect Best Effort

thr_pub_  = node_->create_publisher<px4_msgs::msg::RoverThrottleSetpoint>("/fmu/in/rover_throttle_setpoint", qos_be);

str_pub_  = node_->create_publisher<px4_msgs::msg::RoverSteeringSetpoint>("/fmu/in/rover_steering_setpoint", qos_be);

goto_pub_ = node_->create_publisher<px4_msgs::msg::GotoSetpoint>          ("/fmu/in/goto_setpoint",           qos_be);

cfg_pub_  = node_->create_publisher<px4_msgs::msg::VehicleControlMode>    ("/fmu/in/config_control_setpoints", rclcpp::QoS(10).best_effort());

cmd_pub_  = node_->create_publisher<px4_msgs::msg::VehicleCommand>        ("/fmu/in/vehicle_command", 10);

lpos_ = std::make_shared<px4_ros2::OdometryLocalPosition>(*this);

// timers

tick_timer_ = node_->create_wall_timer(50ms, std::bind(&FlightModeTest::tick, this));         // 20 Hz

cfg_timer_  = node_->create_wall_timer(1s,  std::bind(&FlightModeTest::publishLegacyConfig, this)); // 1 Hz

}

void onActivate() override {

burst_left_ = 10; // ~0.5s extra config bursts (10 * 50ms)

last_log_ns_ = 0;

}

void onDeactivate() override {}

// IMPORTANT: NO multicopter setpoints → keep MC controllers off

void updateSetpoint(float /*dt_s*/) override {

return; // no-op; this avoids flag_control_rates_enabled=true

}

private:

// ---- legacy config heartbeat → enable External+allocation, no other loops ----

void publishLegacyConfig()

{

px4_msgs::msg::VehicleControlMode cfg{};

cfg.timestamp = 0;

cfg.flag_control_manual_enabled    = false;

cfg.flag_control_auto_enabled      = true;   // External counts as "auto"

cfg.flag_control_offboard_enabled  = false;

cfg.flag_control_position_enabled      = false;

cfg.flag_control_velocity_enabled      = false;

cfg.flag_control_altitude_enabled      = false;

cfg.flag_control_climb_rate_enabled    = false;

cfg.flag_control_acceleration_enabled  = false;

cfg.flag_control_attitude_enabled      = false;

cfg.flag_control_rates_enabled         = false;

cfg.flag_control_allocation_enabled = true;

cfg.flag_control_termination_enabled = false;

cfg.source_id = 24; // hint External-2

cfg_pub_->publish(cfg);

// quiet: log every 5s

const auto now = node_->get_clock()->now().nanoseconds();

if (now - last_log_ns_ > 5'000'000'000LL) {

last_log_ns_ = now;

RCLCPP_INFO(rclcpp::get_logger(kName), "External conductor: config heartbeat sent");

}

}

// ---- optional keep-alive: hold at current position (0 speed) ----

void publishGotoHold()

{

const auto ned = lpos_->positionNed();

px4_msgs::msg::GotoSetpoint g{};

g.timestamp = 0;

g.position[0] = ned.x();

g.position[1] = ned.y();

g.position[2] = ned.z();

g.flag_control_heading = false;

g.heading = std::numeric_limits<float>::quiet_NaN();

g.flag_set_max_horizontal_speed = true;  g.max_horizontal_speed = 0.0f;

g.flag_set_max_vertical_speed   = false; g.max_vertical_speed   = 0.0f;

g.flag_set_max_heading_rate     = true;  g.max_heading_rate     = 0.7854f;

goto_pub_->publish(g);

}

// ---- rover outputs (enabled when ENABLE_ROVER_OUTPUTS=1) ----

void publishRover(float thr_norm, float steer_norm)

{

px4_msgs::msg::RoverThrottleSetpoint t{};

t.timestamp = 0;

t.throttle_body_x = clampf(thr_norm, -1.f, 1.f);

t.throttle_body_y = 0.f; // Ackermann only

thr_pub_->publish(t);

px4_msgs::msg::RoverSteeringSetpoint s{};

s.timestamp = 0;

s.normalized_steering_angle = clampf(steer_norm, -1.f, 1.f);

s.normalized_speed_diff     = 0.f; // skid-steer only

str_pub_->publish(s);

}

void sendArm(bool arm)

{

px4_msgs::msg::VehicleCommand cmd{};

cmd.timestamp = 0;

cmd.command = 400; // VEHICLE_CMD_COMPONENT_ARM_DISARM

cmd.param1  = arm ? 1.f : 0.f;

cmd.target_system = 1; cmd.target_component = 1;

cmd.source_system = 1; cmd.source_component = 1;

cmd.from_external = true;

cmd_pub_->publish(cmd);

}

void tick()

{

if (burst_left_ > 0) { --burst_left_; publishLegacyConfig(); } // extra help first 0.5s

#if ENABLE_ROVER_OUTPUTS

// Map keyboard: throttle stick → throttle, ROLL (←/→) → steering

const float thr   = clampf(_manual_control_input->throttle(), -1.f, 1.f);

const float steer = clampf(_manual_control_input->roll(),     -1.f, 1.f);

publishGotoHold();        // keep-alive (0 speed, no motion)

publishRover(thr, steer); // drive rover

#else

(void)_manual_control_input; // silence unused warning

#endif

}

private:

// default example members (we just don't use MC setpoints)

std::shared_ptr<px4_ros2::ManualControlInput> _manual_control_input;

std::shared_ptr<px4_ros2::RatesSetpointType> _rates_setpoint;

std::shared_ptr<px4_ros2::AttitudeSetpointType> _attitude_setpoint;

std::shared_ptr<px4_ros2::PeripheralActuatorControls> _peripheral_actuator_controls;

float _yaw{0.f};

// rover additions

std::shared_ptr<px4_ros2::OdometryLocalPosition> lpos_;

rclcpp::Publisher<px4_msgs::msg::RoverThrottleSetpoint>::SharedPtr thr_pub_;

rclcpp::Publisher<px4_msgs::msg::RoverSteeringSetpoint>::SharedPtr str_pub_;

rclcpp::Publisher<px4_msgs::msg::GotoSetpoint>::SharedPtr          goto_pub_;

rclcpp::Publisher<px4_msgs::msg::VehicleControlMode>::SharedPtr    cfg_pub_;

rclcpp::Publisher<px4_msgs::msg::VehicleCommand>::SharedPtr        cmd_pub_;

rclcpp::TimerBase::SharedPtr tick_timer_;

rclcpp::TimerBase::SharedPtr cfg_timer_;

rclcpp::Node* node_{nullptr};

int    burst_left_{0};

int64_t last_log_ns_{0};

};



Select mode 
Arm 
Run below 
ros2 topic pub -r 5   --qos-reliability best_effort --qos-durability volatile   /fmu/in/config_control_setpoints px4_msgs/msg/VehicleControlMode   "{timestamp: 0,

flag_control_manual_enabled: false,

flag_control_auto_enabled: true,

flag_control_offboard_enabled: false,

flag_control_position_enabled: false,

flag_control_velocity_enabled: false,

flag_control_altitude_enabled: false,

flag_control_climb_rate_enabled: false,

flag_control_acceleration_enabled: false,

flag_control_attitude_enabled: false,

flag_control_rates_enabled: false,

flag_control_allocation_enabled: true,

flag_control_termination_enabled: false,

source_id: 23}"

check Auto is true 

pxlabs@Nilan-PC:~/ros2_ws$ ros2 topic echo --once /fmu/out/vehicle_control_mode

timestamp: 1756324854761472

flag_armed: true

flag_multicopter_position_control_enabled: false

flag_control_manual_enabled: false

flag_control_auto_enabled: true

flag_control_offboard_enabled: false

flag_control_position_enabled: false

flag_control_velocity_enabled: false

flag_control_altitude_enabled: false

flag_control_climb_rate_enabled: false

flag_control_acceleration_enabled: false

flag_control_attitude_enabled: false

flag_control_rates_enabled: false

flag_control_allocation_enabled: true

flag_control_termination_enabled: false

source_id: 23

---

pxlabs@Nilan-PC:~/ros2_ws$
Run Throttle

ros2 topic pub -r 20 --qos-reliability best_effort --qos-durability volatile   /fmu/in/rover_throttle_setpoint px4_msgs/msg/RoverThrottleSetpoint   "{timestamp: 0, throttle_body_x: 0.25, throttle_body_y: 0.0}"

Run Streeing

ros2 topic pub -r 10 --qos-reliability best_effort --qos-durability volatile   /fmu/in/rover_steering_setpoint px4_msgs/msg/RoverSteeringSetpoint   "{timestamp:

0, normalized_steering_angle: 0.8, normalized_speed_diff: 0.0}"

Chatgpt Cli Command

# throttle

ros2 topic pub -r 20 --qos-reliability best_effort --qos-durability volatile \

/fmu/in/rover_throttle_setpoint px4_msgs/msg/RoverThrottleSetpoint \

"{timestamp: 0, throttle_body_x: 0.25, throttle_body_y: 0.0}"

# steering (Ackermann)

ros2 topic pub -r 10 --qos-reliability best_effort --qos-durability volatile \

/fmu/in/rover_steering_setpoint px4_msgs/msg/RoverSteeringSetpoint \

"{timestamp: 0, normalized_steering_angle: 0.8, normalized_speed_diff: 0.0}"

# stop

ros2 topic pub -1 --qos-reliability best_effort --qos-durability volatile \

/fmu/in/rover_throttle_setpoint px4_msgs/msg/RoverThrottleSetpoint \

"{timestamp: 0, throttle_body_x: 0.0, throttle_body_y: 0.0}"



Updated Code with config and registration 


#pragma once

/****************************************************************************

* External Rover Conductor (with config heartbeat)

* - Publishes /fmu/in/config_control_setpoints (AUTO + ALLOC true, others false)

* - Rover throttle/steering only

* - Dummy MC setpoint types created (not published)

****************************************************************************/

#include <px4_ros2/components/mode.hpp>

#include <px4_ros2/odometry/local_position.hpp>

#include <px4_ros2/control/setpoint_types/experimental/rates.hpp>

#include <px4_ros2/control/setpoint_types/experimental/attitude.hpp>

#include <px4_msgs/msg/vehicle_control_mode.hpp>

#include <px4_msgs/msg/register_ext_component_request.hpp>

#include <px4_msgs/msg/register_ext_component_reply.hpp>

#include <px4_msgs/msg/arming_check_request.hpp>

#include <px4_msgs/msg/vehicle_command.hpp>

#include <px4_msgs/msg/rover_throttle_setpoint.hpp>

#include <px4_msgs/msg/rover_steering_setpoint.hpp>

#include <px4_msgs/msg/goto_setpoint.hpp>

#include <rclcpp/rclcpp.hpp>

#include <algorithm>

#include <cstring>

#include <string>

#include <chrono>

#include <limits>

using namespace std::chrono_literals;

static const std::string kModeName = "External Rover Conductor";

static inline float clampf(float v, float lo, float hi) {

return std::min(std::max(v, lo), hi);

}

class FlightModeTest : public px4_ros2::ModeBase

{

public:

explicit FlightModeTest(rclcpp::Node & node)

: ModeBase(node, kModeName), node_(&node)

{

this->setSkipMessageCompatibilityCheck();

// ---- params ----

external_slot_   = declare_param<int>("external_slot", 1);

source_id_       = (external_slot_ == 2) ? 24u : 23u;

heartbeat_hz_    = std::max<int>(1, declare_param<int>("heartbeat_hz", 1));

setpoint_hz_     = std::max<int>(1, declare_param<int>("setpoint_hz", 20));

do_register_     = declare_param<bool>("do_register", true);

reg_arm_check_   = declare_param<bool>("register_arming_check", true);

reg_mode_        = declare_param<bool>("register_mode", true);

reg_mode_exec_   = declare_param<bool>("register_mode_executor", false);

comp_name_       = declare_param<std::string>("component_name", "goto_external_conductor");

enabled_         = declare_param<bool>("enabled", false);

use_goto_keepalive_ = declare_param<bool>("use_goto_keepalive", false);

throttle_x_      = clampf((float)declare_param<double>("throttle_body_x", 0.0), -1.f, 1.f);

throttle_y_      = clampf((float)declare_param<double>("throttle_body_y", 0.0), -1.f, 1.f);

steering_        = clampf((float)declare_param<double>("steering_angle",   0.0), -1.f, 1.f);

speed_diff_      = clampf((float)declare_param<double>("speed_diff",       0.0), -1.f, 1.f);

param_cb_ = node_->add_on_set_parameters_callback(

[this](const std::vector<rclcpp::Parameter> &ps) {

for (const auto &p : ps) {

const auto &n = p.get_name();

if (n == "external_slot")       { external_slot_ = p.as_int(); source_id_ = (external_slot_==2)?24u:23u; }

else if (n == "heartbeat_hz")   { heartbeat_hz_ = std::max<int>(1, (int)p.as_int());  cfg_timer_->reset(); }

else if (n == "setpoint_hz")    { setpoint_hz_  = std::max<int>(1, (int)p.as_int());  tick_timer_->reset(); }

else if (n == "do_register")    { do_register_  = p.as_bool(); }

else if (n == "register_arming_check")  { reg_arm_check_ = p.as_bool(); }

else if (n == "register_mode")          { reg_mode_ = p.as_bool(); }

else if (n == "register_mode_executor") { reg_mode_exec_ = p.as_bool(); }

else if (n == "component_name") { comp_name_ = p.as_string(); }

else if (n == "enabled")        { enabled_ = p.as_bool(); }

else if (n == "use_goto_keepalive") { use_goto_keepalive_ = p.as_bool(); }

else if (n == "throttle_body_x"){ throttle_x_ = clampf(p.as_double(), -1.0, 1.0); }

else if (n == "throttle_body_y"){ throttle_y_ = clampf(p.as_double(), -1.0, 1.0); }

else if (n == "steering_angle") { steering_   = clampf(p.as_double(), -1.0, 1.0); }

else if (n == "speed_diff")     { speed_diff_ = clampf(p.as_double(), -1.0, 1.0); }

}

rcl_interfaces::msg::SetParametersResult r; r.successful = true; return r;

});

// ---- pubs/subs ----

auto be = rclcpp::QoS(10).best_effort();

cfg_pub_  = node_->create_publisher<px4_msgs::msg::VehicleControlMode>    ("/fmu/in/config_control_setpoints", be);

thr_pub_  = node_->create_publisher<px4_msgs::msg::RoverThrottleSetpoint> ("/fmu/in/rover_throttle_setpoint",  be);

str_pub_  = node_->create_publisher<px4_msgs::msg::RoverSteeringSetpoint> ("/fmu/in/rover_steering_setpoint",  be);

goto_pub_ = node_->create_publisher<px4_msgs::msg::GotoSetpoint>          ("/fmu/in/goto_setpoint",            be);

reg_pub_  = node_->create_publisher<px4_msgs::msg::RegisterExtComponentRequest>("/fmu/in/register_ext_component_request", 10);

reg_sub_  = node_->create_subscription<px4_msgs::msg::RegisterExtComponentReply>(

"/fmu/out/register_ext_component_reply", rclcpp::QoS(10).best_effort(),

[this](px4_msgs::msg::RegisterExtComponentReply::SharedPtr){ RCLCPP_INFO(node_->get_logger(),"Got RegisterExtComponentReply"); });

armchk_sub_ = node_->create_subscription<px4_msgs::msg::ArmingCheckRequest>(

"/fmu/out/arming_check_request", rclcpp::QoS(10).best_effort(),

[this](px4_msgs::msg::ArmingCheckRequest::SharedPtr m){

if (!printed_armchk_) { printed_armchk_=true; RCLCPP_DEBUG(node_->get_logger(),"Arming check request (id=%u, only printed once)", m->request_id); }

});

cmd_pub_  = node_->create_publisher<px4_msgs::msg::VehicleCommand>("/fmu/in/vehicle_command", 10);

// ---- timers ----

tick_timer_ = node_->create_wall_timer(std::chrono::milliseconds(1000/std::max(1,setpoint_hz_)), std::bind(&FlightModeTest::tick,this));

cfg_timer_  = node_->create_wall_timer(std::chrono::milliseconds(1000/std::max(1,heartbeat_hz_)), std::bind(&FlightModeTest::publishConfig,this));

// Optional local pos for keepalive

lpos_ = std::make_shared<px4_ros2::OdometryLocalPosition>(*this);

// Dummy setpoint types (required by ModeBase; not used)

_rates_sp_ = std::make_shared<px4_ros2::RatesSetpointType>(*this);

_att_sp_   = std::make_shared<px4_ros2::AttitudeSetpointType>(*this);

}

void onActivate() override

{

RCLCPP_DEBUG(node_->get_logger(), "Registering '%s' (arming check: %d, mode: %d, mode executor: %d)",

comp_name_.c_str(), reg_arm_check_, reg_mode_, reg_mode_exec_);

burst_left_ = 10;

burst_timer_ = node_->create_wall_timer(50ms, std::bind(&FlightModeTest::burstTick, this));

if (do_register_) {

RCLCPP_DEBUG(node_->get_logger(), "Subscriber found, continuing");

RCLCPP_DEBUG(node_->get_logger(), "Publisher found, continuing");

sendRegistration();

} else {

RCLCPP_WARN(node_->get_logger(), "Skipping registration (param do_register=false)");

}

last_cfg_log_ns_ = 0;

RCLCPP_DEBUG(node_->get_logger(), "Mode '%s' activated", kModeName.c_str());

RCLCPP_DEBUG(node_->get_logger(), "Mode '%s': changing setpoint type", kModeName.c_str());

}

void onDeactivate() override {}

void updateSetpoint(float) override { /* no MC publishing */ }

private:

template<typename T> T declare_param(const std::string &name, const T &def){ return node_->declare_parameter<T>(name, def); }

void sendRegistration()

{

px4_msgs::msg::RegisterExtComponentRequest req{};

req.timestamp = 0;

std::fill(req.name.begin(), req.name.end(), 0);

const size_t n = std::min(comp_name_.size(), req.name.size()-1);

std::memcpy(req.name.data(), comp_name_.data(), n);

req.register_arming_check  = reg_arm_check_;

req.register_mode          = reg_mode_;

req.register_mode_executor = reg_mode_exec_;

reg_pub_->publish(req);

}

void publishConfig()

{

px4_msgs::msg::VehicleControlMode m{};

m.timestamp = 0;

m.flag_control_manual_enabled      = false;

m.flag_control_auto_enabled        = true;

m.flag_control_offboard_enabled    = false;

m.flag_control_position_enabled    = false;

m.flag_control_velocity_enabled    = false;

m.flag_control_altitude_enabled    = false;

m.flag_control_climb_rate_enabled  = false;

m.flag_control_acceleration_enabled= false;

m.flag_control_attitude_enabled    = false;

m.flag_control_rates_enabled       = false;

m.flag_control_allocation_enabled  = true;

m.flag_control_termination_enabled = false;

m.source_id = source_id_;

cfg_pub_->publish(m);

const auto now = node_->get_clock()->now().nanoseconds();

if (now - last_cfg_log_ns_ > 5'000'000'000LL) {

last_cfg_log_ns_ = now;

RCLCPP_INFO(node_->get_logger(), "External conductor: config heartbeat sent (source_id=%u)", source_id_);

}

}

void publishGotoKeepAlive()

{

if (!use_goto_keepalive_) return;

const auto ned = lpos_->positionNed();

px4_msgs::msg::GotoSetpoint g{};

g.timestamp = 0;

g.position[0]=ned.x(); g.position[1]=ned.y(); g.position[2]=ned.z();

g.flag_control_heading=false; g.heading=std::numeric_limits<float>::quiet_NaN();

g.flag_set_max_horizontal_speed=true; g.max_horizontal_speed=0.0f;

g.flag_set_max_vertical_speed=false;  g.max_vertical_speed  =0.0f;

g.flag_set_max_heading_rate=true;     g.max_heading_rate    =0.7854f;

goto_pub_->publish(g);

}

void publishRoverSetpoints()

{

if (!enabled_) return;

px4_msgs::msg::RoverThrottleSetpoint t{}; t.timestamp=0; t.throttle_body_x=throttle_x_; t.throttle_body_y=throttle_y_;

px4_msgs::msg::RoverSteeringSetpoint s{}; s.timestamp=0; s.normalized_steering_angle=steering_; s.normalized_speed_diff=speed_diff_;

thr_pub_->publish(t); str_pub_->publish(s);

}

void burstTick()

{

if (burst_left_-- > 0) { publishConfig(); if (enabled_) publishRoverSetpoints(); }

else { burst_timer_->cancel(); }

}

void tick() { publishRoverSetpoints(); publishGotoKeepAlive(); }

private:

rclcpp::Node* node_{nullptr};

int external_slot_{1}; unsigned source_id_{23};

int heartbeat_hz_{1};  int setpoint_hz_{20};

bool do_register_{true}; bool reg_arm_check_{true}; bool reg_mode_{true}; bool reg_mode_exec_{false};

std::string comp_name_{"goto_external_conductor"};

bool enabled_{false}; bool use_goto_keepalive_{false};

float throttle_x_{0.f}, throttle_y_{0.f}, steering_{0.f}, speed_diff_{0.f};

rclcpp::Publisher<px4_msgs::msg::VehicleControlMode>::SharedPtr          cfg_pub_;

rclcpp::Publisher<px4_msgs::msg::RoverThrottleSetpoint>::SharedPtr       thr_pub_;

rclcpp::Publisher<px4_msgs::msg::RoverSteeringSetpoint>::SharedPtr       str_pub_;

rclcpp::Publisher<px4_msgs::msg::GotoSetpoint>::SharedPtr                goto_pub_;

rclcpp::Publisher<px4_msgs::msg::RegisterExtComponentRequest>::SharedPtr reg_pub_;

rclcpp::Subscription<px4_msgs::msg::RegisterExtComponentReply>::SharedPtr reg_sub_;

rclcpp::Subscription<px4_msgs::msg::ArmingCheckRequest>::SharedPtr       armchk_sub_;

rclcpp::Publisher<px4_msgs::msg::VehicleCommand>::SharedPtr              cmd_pub_;

std::shared_ptr<px4_ros2::OdometryLocalPosition> lpos_;

rclcpp::TimerBase::SharedPtr tick_timer_, cfg_timer_, burst_timer_;

int burst_left_{0}; int64_t last_cfg_log_ns_{0}; bool printed_armchk_{false};

std::shared_ptr<px4_ros2::RatesSetpointType>    _rates_sp_;

std::shared_ptr<px4_ros2::AttitudeSetpointType> _att_sp_;

rclcpp::node_interfaces::OnSetParametersCallbackHandle::SharedPtr param_cb_;

};



Main.CPP

#include "rclcpp/rclcpp.hpp"

#include <px4_ros2/components/node_with_mode.hpp>

#include <string>

#include "../include/mode.hpp"

using MyNodeWithMode = px4_ros2::NodeWithMode<FlightModeTest>;

static const std::string kNodeName = "example_mode_goto";

static const bool kEnableDebugOutput = true;

int main(int argc, char* argv[]) {

rclcpp::init(argc, argv);

rclcpp::spin(std::make_shared<MyNodeWithMode>(kNodeName, kEnableDebugOutput));

rclcpp::shutdown();

return 0;

}

Then Run 

ros2 run example_mode_goto_cpp example_mode_goto \

--ros-args -p external_slot:=1 -p heartbeat_hz:=1 -p setpoint_hz:=20 \

-p do_register:=true -p enabled:=false


or 

ros2 run example_mode_goto_cpp example_mode_goto \

--ros-args -p external_slot:=1 -p heartbeat_hz:=5 -p setpoint_hz:=20 \

-p do_register:=true -p enabled:=false


and then below 
ros2 param set /example_mode_goto enabled true

Enable your node’s rover outputs

ros2 param set /example_mode_goto enabled true

A) Set parameters on the node (nice for testing)

# throttle forward 0.25

ros2 param set /example_mode_goto throttle_body_x 0.25

# steer right 0.4 (Ackermann: -1 left … +1 right)

ros2 param set /example_mode_goto steering_angle 0.4

# stop + center

ros2 param set /example_mode_goto throttle_body_x 0.0

ros2 param set /example_mode_goto steering_angle 0.0